MASTERING POINTERS IN C

Volume II

THE DEEP DIVE

Hardware mechanics, the C standard, undefined behavior, and what the compiler actually does

Read this AFTER Volume I.

Volume I gives you working knowledge.

Volume II gives you understanding.

# How To Read This Volume

This book is dense. It assumes you've worked through Volume I and can write basic pointer code without crashing. Now you're going to learn why everything works the way it does — at the level of bytes, registers, and the C standard.

Pace yourself. A single chapter may take a week of careful study. The reward is that pointers will stop being mysterious — they'll feel mechanical, predictable, and obvious.

### Recommended Approach

- Read each chapter once for the big idea.

- Type out every code example by hand. Run it. Modify it. Break it on purpose.

- Re-read the chapter until the diagrams in your head match the diagrams on the page.

- Do the exercises. Don't peek at solutions until you've sat with each problem for at least 20 minutes.

- Move on only when you can explain the chapter to someone else without notes.

### The Sixteen Chapters

| Chapter | Topic |
| --- | --- |
| Ch 1 | What an Address Really Is — Hardware up to C |
| Ch 2 | How Variables Become Addresses — The Compiler's Job |
| Ch 3 | The Stack Frame in Microscopic Detail |
| Ch 4 | Reading Pointer Declarations Like a Compiler |
| Ch 5 | Pointer Arithmetic — The Standard's Exact Rules |
| Ch 6 | Arrays vs. Pointers — Where the Equivalence Breaks |
| Ch 7 | L-values, R-values, and Why *p++ Works |
| Ch 8 | Strings — Encoding, Storage, and the Const Pitfall |
| Ch 9 | Calling Conventions — How Pointers Move Between Functions |
| Ch 10 | The Heap — How malloc Actually Works Inside |
| Ch 11 | Alignment, Padding, and Why It Matters |
| Ch 12 | Aliasing, restrict, and the Strict Aliasing Rule |
| Ch 13 | void*, Type Punning, and Generic Programming |
| Ch 14 | Function Pointers and the ABI |
| Ch 15 | Undefined Behavior — A Field Guide |
| Ch 16 | Mastery Drills — 50 Problems With Worked Solutions |

# Chapter 1 — What an Address Really Is

Before C, before pointers, there is the hardware. To understand pointers, you must understand what an address means to the CPU. Let's start at the bottom.

## 1.1 RAM Is a Numbered Sequence of Bytes

Random Access Memory (RAM) is, physically, billions of tiny capacitor cells, each storing one bit. The hardware groups them into bytes — 8 bits each. Every byte is given a unique number, called its physical address, starting from 0 and going up to whatever your machine supports.

If your machine has 16 GB of RAM, then there are 16 × 1024 × 1024 × 1024 ≈ 17 billion bytes, and addresses range from 0 to about 17,179,869,183. To represent that range in binary, you need at least 34 bits — which is why your machine has at least 64-bit addresses (we round up for hardware convenience).

| Address | Contents | Variable |
| --- | --- | --- |
| 0x00000000 | 0x42 | byte 0 |
| 0x00000001 | 0xFF | byte 1 |
| 0x00000002 | 0x07 | byte 2 |
| 0x00000003 | 0x00 | byte 3 |
| ... | ... | ... |
| 0xFFFFFFFE | 0x10 | second-to-last |
| 0xFFFFFFFF | 0x33 | last byte (4 GB machine) |

That's it. RAM is just a big array. The address is the index. The contents at that address are 8 bits — one byte — interpreted as whatever the program needs (an integer, a character, part of a float, an instruction).

## 1.2 The CPU Doesn't Know What Anything "Is"

This is the most important sentence in this entire volume:

THE CENTRAL TRUTH The CPU sees memory as undifferentiated bytes. It does not know which bytes are integers, which are pointers, which are characters, which are floats, or which are instructions. The interpretation comes from the program's instructions, which the CPU executes one at a time without ever 'understanding' the data.

When the CPU executes an ADD instruction, it reads bytes from a register, treats them as an integer, and adds. When it executes a FADD (floating-point add), it reads the same kind of bytes and treats them as a float. The bytes themselves don't change identity — only the operation gives them meaning.

This is why C requires you to declare types: not because the hardware needs them, but because the compiler does. The compiler uses your type declarations to choose which machine instructions to emit.

## 1.3 Virtual Memory — A Layer of Lies

Modern operating systems don't let your program see physical addresses. Instead, every process gets its own virtual address space — an illusion that it owns all of memory, from address 0 up to some huge number.

When your program reads or writes an address, the CPU's Memory Management Unit (MMU) translates the virtual address to a physical address using a structure called the page table. Different processes have different page tables, so their virtual addresses map to different physical bytes.

🔬 DEEPER Why bother? Three reasons. (1) Isolation: process A cannot read process B's memory because A's virtual addresses don't map to B's physical pages. (2) Convenience: every program can pretend it owns memory starting at address 0, simplifying compilation and linking. (3) Larger-than-RAM working sets: rarely-used pages can be swapped to disk while the program runs. When you print a pointer in C, you're printing a virtual address — meaningful only inside your process. The same virtual address in another process points to entirely different physical memory.

## 1.4 Why Addresses Are 64 Bits Even For a Single Char

On a 64-bit machine, every pointer — `int*`, `char*`, `double*`, function pointers — is 8 bytes wide. The pointer's size doesn't depend on what it points to. It depends on the address space's size.

A pointer is just an address. Addresses are numbers. On a 64-bit machine, the CPU's address registers are 64 bits wide, so addresses are 64 bits. That's all.

#include <stdio.h> int main(void) { printf("sizeof(char*) = %zu\n", sizeof(char*)); printf("sizeof(int*) = %zu\n", sizeof(int*)); printf("sizeof(double*) = %zu\n", sizeof(double*)); printf("sizeof(void(*)())= %zu\n", sizeof(void(*)())); return 0; } // On a 64-bit machine, all four print 8.

📌 KEY POINT On embedded systems with 16-bit microcontrollers, all pointers are 2 bytes. On old 32-bit x86, all pointers are 4 bytes. The rule is the same: pointer size matches address-bus size.

## 1.5 The First Mental Model You Need

Forget pointers for a moment. Picture this:

- A long row of mailboxes, each numbered.

- Each mailbox holds a single byte (a number from 0 to 255).

- Some mailboxes hold parts of a larger object — like a 4-byte integer occupying mailboxes 100, 101, 102, 103.

- A pointer is a piece of paper that has a mailbox number written on it.

- To read what's at the pointer, you go to that mailbox and look inside.

- To write through the pointer, you go to that mailbox and put something inside.

- The CPU is a worker who only knows how to read mailbox numbers, fetch bytes, and put bytes back.

Hold this picture firmly. Every pointer concept in this book is just elaboration on this image.

# Chapter 2 — How Variables Become Addresses

## 2.1 The Compiler Is a Translator With a Notebook

When you write `int x = 5;`, the compiler does something concrete and mechanical. It is not magic. Watch:

- It picks an unused address — say, 0x7ffe0010 — to store x.

- It writes in its symbol table: "the name 'x' refers to the 4 bytes starting at 0x7ffe0010."

- It emits machine code that, when executed, will write the value 5 into those 4 bytes.

- After x is declared, every later use of the name x is replaced (by the compiler) with the address 0x7ffe0010.

The variable name is purely a compile-time concept. By the time the program is running, the names are gone. There's no "x" in memory — just bytes at an address that the running code knows to access.

## 2.2 What This Looks Like in Assembly

Let's compile a tiny C program and look at the actual machine instructions. Here's the C:

int main(void) { int x = 5; int y = x + 3; return y; }

Compiled with `gcc -O0 -S` on x86-64 Linux, the assembly (cleaned up) looks roughly like this:

main: push %rbp ; save old base pointer mov %rsp, %rbp ; set new base pointer movl $5, -8(%rbp) ; write 5 into [rbp-8] ← this is x movl -8(%rbp), %eax ; load x into register addl $3, %eax ; add 3 movl %eax, -4(%rbp) ; store result into [rbp-4] ← this is y movl -4(%rbp), %eax ; return y pop %rbp ret

Notice: the names x and y don't appear in the assembly. The compiler has replaced them with offsets from rbp (the base pointer register). x lives at rbp-8, y at rbp-4. The variables are just locations; the names existed only in your source code.

🔬 DEEPER When you write &x in C, the compiler emits code that computes 'rbp - 8' and stores the result. That's all the address-of operator does — it asks the compiler to give you the location it has assigned to a variable, instead of the value at that location.

## 2.3 Storage Duration — Where Variables Live

The C standard defines four storage durations. Each determines where the variable lives, when it comes into existence, and when it dies.

| Duration | Lifetime | Examples |
| --- | --- | --- |
| Static | Whole program | Globals, static locals, string literals |
| Automatic | Function call to return | Regular local variables |
| Allocated | malloc() to free() | Heap memory |
| Thread (C11) | Thread start to thread exit | _Thread_local variables |

This is why returning the address of a local variable is broken — its automatic lifetime ends the moment the function returns. The address still exists, but it now refers to memory that's been reclaimed and may be reused by the next function call.

## 2.4 The Symbol Table Lives Inside the Compiler

During compilation, the symbol table maps names to information about each variable: type, storage class, address (or stack offset), scope, etc. After compilation, the symbol table is mostly thrown away — though debug builds keep a copy so debuggers can show you variable names.

# Compile with debug info gcc -g program.c -o program # The compiler still writes a symbol table to the binary, # in DWARF format. You can inspect it: objdump --dwarf=info program ¦ grep -A3 "DW_AT_name.*\"x\"" # In a release build (no -g), names are gone: gcc -O2 program.c -o program # x and y as names exist nowhere in the binary.

## 2.5 Putting It Together

Here's the chain of meaning, from your code to running hardware:

- You type a name. The compiler keeps a notebook entry for it.

- The compiler picks an address (or stack offset) for the variable.

- Every time you use the name, the compiler emits machine code that reads or writes that address.

- The CPU executes those instructions, accessing physical memory through virtual addresses translated by the MMU.

- The variable is, ultimately, a region of physical RAM bytes. Nothing more.

A pointer is a variable whose value happens to be one of those addresses. That's why pointers feel circular at first — they are. A pointer is an address, but the pointer itself is stored at an address, and you can have a pointer to that address (a pointer-to-pointer), and so on.

# Chapter 3 — The Stack Frame in Microscopic Detail

## 3.1 The Call Stack Is a Stack Data Structure in RAM

When function A calls function B, the CPU needs to remember (a) where in A to return when B finishes, (b) the values of registers A was using, and (c) where to store B's local variables. The call stack handles all three.

On x86-64 Linux, the stack lives in high memory and grows downward (toward smaller addresses). Two registers manage it: rsp (stack pointer, points to the current top) and rbp (base pointer, points to the start of the current function's frame).

## 3.2 Anatomy of a Stack Frame

A stack frame is the region of stack memory that belongs to a single function call. Here's what's in one, from high to low addresses:

| Address | Contents | Variable |
| --- | --- | --- |
| rbp + 16+ | (caller's args, if any) | ↑ caller's frame |
| rbp + 8 | return address | set by CALL |
| rbp + 0 | saved old rbp | set by 'push rbp' |
| rbp - 4 | first local var | this function's frame |
| rbp - 8 | second local var |  |
| rbp - 16 | third local var |  |
| rsp | top of stack → | next function's frame begins below |

Each function call pushes a new frame. Each return pops one. The whole structure looks like a stack of plates, where each plate is one function's working memory.

## 3.3 What Happens On Function Call (Step By Step)

Suppose main() calls foo(5, 10). On x86-64 (System V ABI):

- Arguments are placed in registers (rdi=5, rsi=10) — the first six integer args go in registers.

- CALL instruction pushes the return address onto the stack and jumps to foo's code.

- foo's prologue runs: `push rbp` (saves caller's frame pointer), `mov rsp, rbp` (sets up own frame pointer).

- foo's prologue then `sub rsp, N` to make room for local variables — N is computed at compile time.

- foo's body executes. Locals at rbp-4, rbp-8, etc.

- foo's epilogue runs: `mov rbp, rsp` (restore stack to frame start), `pop rbp` (restore caller's frame pointer).

- RET pops the return address and jumps back to main.

After RET, foo's frame is gone. The memory it occupied is still physically there, but it's now "above" rsp and considered free — the next function call will reuse those bytes.

⚠ DANGER This is why returning a pointer to a local variable is broken. The local's address is rbp-4 (or wherever) at the moment of the return statement. The pointer you return is that address. But by the time the caller dereferences it, a new function may have been called that put its own data in that exact spot.

## 3.4 A Worked Example

Let's trace through a real call:

int square(int x) { int y = x * x; return y; } int main(void) { int a = 5; int b = square(a); return b; }

Stack state during square() execution (sample addresses, abbreviated):

| Address | Contents | Variable |
| --- | --- | --- |
| 0x7fff0050 | (higher memory) | main's caller's frame |
| 0x7fff0040 | main's saved rbp | main's frame |
| 0x7fff003C | 5 | main's variable a |
| 0x7fff0038 | ? | main's variable b (uninit) |
| 0x7fff0030 | 0x40031A | return addr (back to main) |
| 0x7fff0028 | 0x7fff0040 | saved rbp from main |
| 0x7fff0024 | 5 | square's parameter x |
| 0x7fff0020 | 25 | square's variable y |

When square returns, rsp moves up to 0x7fff0040 again. The bytes at 0x7fff0020 and below are still 25, x, etc. — but they're no longer protected. The next function call will overwrite them.

## 3.5 Why Stack Overflow Crashes

The stack has finite size — typically 1 MB to 8 MB. Each function call consumes some of it. Deep recursion eats the stack alive:

int recurse(int n) { char buf[1024]; // 1 KB per call return recurse(n+1); // never returns } // After ~8000 calls, stack is exhausted. // CPU tries to write past the stack boundary. // MMU sees an access to an unmapped page → SIGSEGV (segfault).

This is why the OS sets a guard page at the bottom of the stack — to ensure stack overflow becomes a clean crash instead of silently corrupting other memory.

## 3.6 Pointers Into the Stack

Local variables have addresses. You can take them with &. You can pass them around. As long as the function whose frame contains them is still active, those addresses remain valid:

void caller(void) { int x = 42; callee(&x); // valid — caller's frame still exists } // when callee runs, it can safely use &x void callee(int *p) { *p = 100; // OK — modifies caller's x }

This works because the call stack guarantees that when callee runs, caller's frame is still on the stack — caller is paused, not finished. The pointer is valid for the duration of the call.

# Chapter 4 — Reading Pointer Declarations

Some C declarations look insane. `int *(*(*foo)(int))[5]` — what is that? You will encounter such declarations in real code. There is a method to read any of them.

## 4.1 The Spiral Rule

Start at the variable name. Spiral outward, reading from the innermost group:

- Begin at the identifier.

- Look right, then left, alternating, expanding outward.

- Read each symbol as you encounter it: `*` = "pointer to", `[]` = "array of", `()` = "function returning".

- Stop when you've consumed everything.

## 4.2 Worked Examples — From Easy to Insane

#### Example 1: int *p

Spiral: start at p. Go left → `*` = "pointer to". Go further left → `int`.

Result: "p is a pointer to int."

#### Example 2: int *p[10]

Spiral: start at p. Go right → `[10]` = "array of 10". Go left → `*` = "pointer to". Go left → `int`.

Result: "p is an array of 10 pointers to int."

#### Example 3: int (*p)[10]

Spiral: start at p. Look at parens — `*` is grouped with p. So → "pointer to". Now leave the parens, go right → `[10]` = "array of 10". Go left → `int`.

Result: "p is a pointer to an array of 10 ints."

THE PARENS RULE Without parens, [] and () bind tighter than *. So `int *p[10]` means `int *(p[10])` — array of pointers. Adding parens around (*p) flips the meaning to a pointer to an array. This single distinction trips up almost everyone the first time.

#### Example 4: int *f(int)

Spiral: start at f. Go right → `(int)` = "function taking int". Now go left → `*` = "returning pointer to". Go left → `int`.

Result: "f is a function taking int, returning pointer to int."

#### Example 5: int (*f)(int)

Spiral: start at f. Parens group `*f` → "pointer to". Leave parens, go right → `(int)` = "function taking int". Go left → `int`.

Result: "f is a pointer to function taking int, returning int."

#### Example 6 (the boss): int *(*(*foo)(int))[5]

Spiral, slowly:

- Start at foo.

- Innermost parens: `(*foo)` → "foo is a pointer to"

- Next, going right: `(int)` → "function taking int"

- Next, going left: `*` → "returning pointer to"

- Next, going right: `[5]` → "an array of 5"

- Next, going left: `*` → "pointers to"

- Finally: `int` → "int"

Result: "foo is a pointer to a function that takes an int and returns a pointer to an array of 5 pointers to int."

💡 INSIGHT There's a Unix command-line tool called cdecl that translates C declarations into English and back. Install it: `apt install cdecl`. Use it to check yourself.

## 4.3 typedef Saves Sanity

In real code, complicated types should always be wrapped in typedef. Compare:

// Without typedef: int *(*(*foo)(int))[5]; // With typedef: typedef int *IntPtr; typedef IntPtr ArrayOf5IntPtrs[5]; typedef ArrayOf5IntPtrs *PtrToArrayOf5IntPtrs; typedef PtrToArrayOf5IntPtrs (*FooFunc)(int); FooFunc foo;

The typedef version is verbose, but every step is comprehensible. This is how mature C libraries (POSIX, Win32, the kernel) declare complex types.

## 4.4 Function Pointers Specifically

The most common confusing declaration is the function pointer. Master this one:

return_type (*pointer_name)(parameter_types); // Examples: int (*p)(void); // pointer to function returning int, taking nothing void (*p)(int); // pointer to function returning void, taking int double (*p)(double, double); // taking two doubles, returning double int (*ops[10])(int, int); // array of 10 such pointers int (*(*p)(int))(int); // pointer to function taking int, returning // pointer to function taking int returning int // With typedef: typedef int (*BinaryIntOp)(int, int); BinaryIntOp ops[10]; // much clearer

# Chapter 5 — Pointer Arithmetic, Exact Rules

The C standard is precise about what pointer arithmetic is legal and what isn't. Casual programmers get away with violations because compilers are lenient — until one day they aren't, and the program corrupts data or crashes after a compiler upgrade.

## 5.1 The Standard's Definition

From C17 §6.5.6 (paraphrased):

RULE FROM THE STANDARD When an integer is added to or subtracted from a pointer, the result has the type of the pointer operand. If the pointer operand points to an element of an array object, and the result points to an element of the same array object (or one past the last element), the result is well-defined. Otherwise, the behavior is undefined.

Three crucial details:

- Pointer arithmetic is defined only WITHIN AN ARRAY (or one past the end).

- Going more than one past the end is undefined, even without dereferencing.

- Going before the start is undefined, even without dereferencing.

## 5.2 What "Inside an Array" Means

A standalone variable counts as an array of length 1. So you can do:

int x = 5; int *p = &x; // p points to a 1-element array int *q = p + 1; // OK — one past the end int *r = p + 2; // UNDEFINED BEHAVIOR int v = *q; // UNDEFINED — even q itself is OK to compute, // but dereferencing one-past-the-end is UB

## 5.3 Why Compilers Take UB Seriously

This isn't theoretical. Modern optimizing compilers exploit pointer-arithmetic UB. Consider:

int arr[10]; int *p = arr; for (int i = 0; ; i++) { if (p + i < arr) break; // can this ever be true? if (p + i >= arr + 10) break; arr[i] = 0; }

A modern compiler may delete the first `break` entirely. Why? Because for the first comparison to make sense, p+i must be a valid pointer in arr (or one past). Inside arr, p+i ≥ arr is always true. The compiler concludes the comparison is always false and removes the dead branch. If your code relies on UB to make that comparison meaningful, the loop becomes infinite.

🔬 DEEPER This is why 'undefined behavior' isn't 'whatever the CPU does.' It's 'the compiler may assume this doesn't happen.' Optimizers use UB as input — they reason 'since this can't happen, I can prove these other things, and emit smaller/faster code.' If your program triggers UB, the optimizer's assumptions are wrong, and the resulting code can do anything: skip lines, run them out of order, infinite-loop, format your hard drive. (The last one is rhetoric, but only barely.)

## 5.4 Pointer Subtraction

Subtracting two pointers gives the number of elements between them — but only if they're in the same array. The result has type ptrdiff_t (defined in <stddef.h>):

int arr[100]; int *p = &arr[10]; int *q = &arr[35]; ptrdiff_t diff = q - p; // 25 — number of ELEMENTS, not bytes // Subtracting pointers from different arrays is UB. int a[10], b[10]; ptrdiff_t bad = &a[0] - &b[0]; // UNDEFINED

## 5.5 Pointer Comparisons

== and != are always defined for pointers, even when they point to different objects (the result is just "not equal"). But <, <=, >, >= are only defined for pointers within the same array:

int a[10], b[10]; int *pa = a, *pb = b; if (pa == pb) ... // OK — always false here, but defined if (pa < pb) ... // UB — pointers to different arrays

## 5.6 The Most Common Idiom

These two loops are equivalent and idiomatic:

int arr[10] = { ... }; // Index-based for (int i = 0; i < 10; i++) { arr[i] = process(arr[i]); } // Pointer-based for (int *p = arr; p < arr + 10; p++) { *p = process(*p); } // arr + 10 is the legal one-past-the-end pointer. // We compare to it but never dereference it.

## 5.7 Bytes vs. Elements — A Common Confusion

If you really need to do byte-level pointer arithmetic, cast to char* first. Pointer arithmetic on char* moves one byte at a time:

int arr[10]; int *p = arr; char *cp = (char*)arr; p + 1 → arr + 4 bytes (one int forward) cp + 1 → arr + 1 byte (one byte forward) cp + 4 → arr + 4 bytes (same as p + 1)

This is sometimes useful when implementing memcpy or working with serialized data, but rarely needed in everyday code.

# Chapter 6 — Arrays vs. Pointers

In Volume I you learned that arrays "decay" to pointers. That's a useful first approximation, but it papers over real differences. Now we go deeper.

## 6.1 The Definition: An Array Is Not a Pointer

An array is a contiguous block of objects of the same type. Its name, in most contexts, is converted to a pointer to its first element. But the name is not a pointer variable — there's no separate storage holding an address. The array name is the storage.

| Address | Contents | Variable |
| --- | --- | --- |
| 0x1000 | 10 | arr[0] |
| 0x1004 | 20 | arr[1] |
| 0x1008 | 30 | arr[2] |
| 0x100C | 40 | arr[3] |

There's no separate "arr pointer" anywhere. The name "arr" exists only at compile time, as a label for the address 0x1000.

Compare to:

| Address | Contents | Variable |
| --- | --- | --- |
| 0x1000 | 10 | arr[0] |
| 0x1004 | 20 | arr[1] |
| 0x1008 | 30 | arr[2] |
| 0x100C | 40 | arr[3] |
| 0x2000 | 0x1000 | int *p = arr; |

Now there's an extra 8-byte storage at 0x2000 that holds the address 0x1000. p is a real variable; arr is not.

## 6.2 The Three Decay Exceptions, Explained

### Exception 1: sizeof

int arr[10]; int *p = arr; sizeof(arr) → 40 (10 ints × 4 bytes) sizeof(p) → 8 (size of a pointer on 64-bit)

sizeof asks the compiler about the type. The type of arr is int[10] — total size 40. The type of p is int* — size 8. The compiler knows this without running the program.

### Exception 2: &arr

int arr[10]; int *p1 = arr; // type int* — points to first element int (*p2)[10] = &arr; // type int(*)[10] — points to whole array p1 + 1 // advances by 1 int (4 bytes) p2 + 1 // advances by 1 array (40 bytes) (void*)p1 == (void*)p2 // TRUE — same numerical address // but the TYPES are different

&arr and arr have the same numerical value (the address of the first element coincides with the address of the array). But their types differ, which means pointer arithmetic on them differs.

### Exception 3: String Literal Initializer

char a[] = "hello"; // copies 6 chars onto stack/static, a is array char *b = "hello"; // b points to read-only string literal a[0] = 'H'; // OK — modifying my own copy b[0] = 'H'; // UB — usually segfault

## 6.3 What Happens in Function Parameters

All three of these declarations mean exactly the same thing:

void f(int *arr); void f(int arr[]); void f(int arr[10]); // the 10 is documentation only — IGNORED

In function parameters, an array type is silently converted to a pointer type. The size, if given, is ignored. This is why you must pass length separately:

void print(int arr[], int n) { // arr is really int* for (int i = 0; i < n; i++) printf("%d ", arr[i]); } int main(void) { int data[5] = {1, 2, 3, 4, 5}; print(data, 5); // must pass 5 explicitly // sizeof(data)/sizeof(data[0]) works in main, not in print }

⚠ DANGER C11 introduced 'static' size declarations: `void f(int arr[static 10])` is a hint that arr must point to at least 10 ints. The compiler may use this for diagnostics, but it doesn't change the parameter type — arr is still int* inside the function.

## 6.4 Multidimensional Arrays — Where It Gets Tricky

int matrix[3][4]; // 3 rows × 4 cols, all in one block // 12 ints total, contiguous in memory // Memory layout: // row 0: matrix[0][0..3] ← addresses 0..15 // row 1: matrix[1][0..3] ← addresses 16..31 // row 2: matrix[2][0..3] ← addresses 32..47 matrix // type: int(*)[4] — pointer to array of 4 ints matrix[0] // type: int* — pointer to first int &matrix[0][0] // type: int* — same numerical addr as above &matrix // type: int(*)[3][4] — pointer to whole matrix matrix[i][j] // *(*( matrix + i ) + j) // = *( matrix[i] + j ) // matrix + i advances by 4 ints (one row) // matrix[i] + j advances by 1 int

Notice: when you write matrix[i][j], the compiler knows matrix has 4 columns because of its declared type. That's why a 2D array as a parameter must be declared with the inner dimension:

void f(int m[][4], int rows); // OK — compiler knows row stride void f(int m[3][4]); // OK — first dim ignored void f(int **m); // DIFFERENT TYPE — array of ptrs to ptrs // You CANNOT pass int[3][4] to a function expecting int**. // They are different types with different memory layouts.

🔬 DEEPER A true 2D array `int m[3][4]` is one contiguous 48-byte block. An 'array of pointers' `int *m[3]` is 3 pointers, each pointing to a separately-allocated row. They look similar in code (both use m[i][j]), but the underlying memory and the access patterns differ entirely. Knowing which you have matters for performance (cache locality) and for malloc/free patterns.

# Chapter 7 — L-values, R-values, and *p++

## 7.1 The L-value / R-value Distinction

Every C expression is either an lvalue or an rvalue. The names come from "left value" and "right value" — what can appear on the left or right side of an assignment.

An lvalue refers to an object that has a location in memory. You can take its address with &. You can assign to it (unless it's const).

An rvalue is just a value — a temporary, a literal, the result of an arithmetic expression. It has no address; you can't take & of it.

| Expression | Kind | Why |
| --- | --- | --- |
| x | lvalue | x is a named variable with an address |
| x + 1 | rvalue | result of arithmetic — temporary |
| arr[i] | lvalue | refers to a specific element in memory |
| *p | lvalue | refers to whatever p points to |
| &x | rvalue | the address itself, computed; you can't &(&x) |
| 5 | rvalue | literal — has no address |
| p + 1 | rvalue | computed pointer — temporary |
| p++ | rvalue | result is the OLD value of p — temporary |

This is why you can write `*p = 5` (lvalue on the left) but not `5 = x` (literal as lvalue) or `x + 1 = 7` (rvalue as lvalue).

## 7.2 Decoding *p++

This is the most famous idiom in C. Let's decode it precisely.

Operator precedence and associativity:

- Postfix `++` has higher precedence than prefix `*`.

- So `*p++` parses as `*(p++)`.

- `p++` is the postfix increment — its VALUE is the old p; its SIDE EFFECT is to increment p.

- So `*p++` means: take the old value of p, dereference it, and (as a side effect) increment p.

char *p = "hello"; char c = *p++; // c gets 'h', p now points to 'e' char d = *p++; // d gets 'e', p now points to 'l' // This is why the classic strcpy is so compact: while ((*dst++ = *src++)) ; // ↑ ↑ ↑ // assign copy and post-increment both pointers // the result of the assignment is the char copied; // when it's '\0' (zero), the while loop ends.

## 7.3 The Four Variants — Tabulated

| Expression | Meaning | After |
| --- | --- | --- |
| *p++ | Read *p, then advance p | p moved one |
| *++p | Advance p, then read *p | p moved one |
| (*p)++ | Read *p, then increment what p points to | p unchanged |
| ++*p | Increment what p points to, then read it | p unchanged |

int arr[3] = {10, 20, 30}; int *p = arr; int a = *p++; // a=10, p→arr[1] int b = *++p; // b=30, p→arr[2] (started at arr[1], advanced to arr[2]) p = arr; int c = (*p)++; // c=10, arr[0] becomes 11, p still at arr[0] p = arr; int d = ++*p; // d=12 (arr[0] was 11), arr[0] becomes 12, p unchanged

## 7.4 The Sequence Point Trap

Modifying the same object twice between sequence points is undefined. So:

int i = 0; int arr[5]; arr[i] = i++; // UNDEFINED BEHAVIOR // i is read AND modified — order unspecified int *p = arr; *p++ = *p; // UNDEFINED BEHAVIOR // p is read on the right and modified on the left int j = i++ + i++; // UNDEFINED — modifying i twice between sequence points

In C11, the rules use "sequenced before" instead of "sequence points," but the conclusion is the same: don't modify a variable more than once in a single expression unless an explicit sequence point separates the modifications.

# Chapter 8 — Strings: Encoding, Storage, the Const Pitfall

## 8.1 What a String Literal Really Is

When you write `"hello"` in C source code, the compiler:

- Allocates 6 bytes in the read-only data segment of the executable: 'h', 'e', 'l', 'l', 'o', '\0'.

- Treats the literal in your code as the address of those bytes.

- If two source files both have `"hello"`, the compiler/linker may combine them into a single shared 6-byte block (called string pooling).

| Address | Contents | Variable |
| --- | --- | --- |
| 0x400600 | 'h' | string literal: "hello" |
| 0x400601 | 'e' | (in .rodata segment) |
| 0x400602 | 'l' | READ-ONLY |
| 0x400603 | 'l' |  |
| 0x400604 | 'o' |  |
| 0x400605 | 0 | null terminator |

## 8.2 Why char *p = "hello" Is Risky

char *p = "hello"; // The compiler creates "hello" in read-only memory. // p holds the address of that read-only block. p[0] = 'H'; // ATTEMPTS to write to read-only memory. // On Linux: SIGSEGV. // On some embedded systems: silently does nothing. // In all cases: UNDEFINED BEHAVIOR.

In C++, this declaration is a compile error. In C, it's allowed for legacy compatibility, but every modern compiler can warn:

gcc -Wwrite-strings program.c # warns on this case // Best practice: always declare string-literal pointers as const. const char *p = "hello"; // now compiler will block writes p[0] = 'H'; // compile error — good

## 8.3 char arr[] = "hello" — What's Different

char arr[] = "hello"; // The compiler: // 1. Computes that the literal needs 6 bytes. // 2. Allocates 6 bytes on the stack (or in writable static memory). // 3. Copies 'h','e','l','l','o','\0' into them at runtime. // arr is now a writable copy of the literal. arr[0] = 'H'; // OK — modifying my own array, not the literal

🔬 DEEPER When this declaration appears at file scope, the compiler may put arr in static memory and emit no copy code at all — the data is initialized when the program loads. Either way, you get a writable buffer, distinct from the read-only literal in .rodata.

## 8.4 Strings In Memory — Visualizing the Difference

After:

char a[] = "hi"; char *b = "hi";

Memory looks like:

| Address | Contents | Variable |
| --- | --- | --- |
| 0x400500 (.rodata) | 'h' 'i' 0 | the literal — read-only |
| 0x7fff0000 (stack) | 'h' 'i' 0 | a — writable copy |
| 0x7fff0008 (stack) | 0x400500 | b — points to literal |

a's address is 0x7fff0000 — pointing into the stack copy. b's value is 0x400500 — pointing into the read-only literal. a[0] writes its own copy. b[0] tries to write the read-only literal.

## 8.5 String Pooling and Identity

const char *a = "hello"; const char *b = "hello"; if (a == b) puts("same address"); // usually YES (string pooling) // But never RELY on this — the standard does not guarantee it. // To compare strings, use strcmp: if (strcmp(a, b) == 0) puts("same content");

## 8.6 Multi-byte and Wide Characters

A C string is a sequence of `char` (typically 1 byte). For ASCII, that's enough. For Unicode, you have options:

| Type | What it is | Used for |
| --- | --- | --- |
| char * | Bytes — interpretation up to you | ASCII, UTF-8 (modern best practice) |
| wchar_t * | Wide chars — 2 or 4 bytes (impl-defined) | Legacy Unicode (Windows uses 2-byte, Linux 4-byte) |
| char16_t * | Exactly 2 bytes (C11) | UTF-16 |
| char32_t * | Exactly 4 bytes (C11) | UTF-32 |

UTF-8 is the modern default: it uses 1-4 bytes per code point, ASCII-compatible (any ASCII string is also valid UTF-8), and works with normal char* and the regular string functions for most operations. Just remember strlen returns BYTES, not characters.

# Chapter 9 — Calling Conventions

## 9.1 What a Calling Convention Is

When function A calls function B, somebody has to specify: where do arguments go? Where does the return value go? Who saves which registers? Where are variadic arguments stored? The answer to these questions is the calling convention — and it's part of the platform's Application Binary Interface (ABI).

The calling convention is not part of the C standard. It's defined by the OS/architecture combination. This is why a compiled .o file from one OS won't link with another's.

## 9.2 System V AMD64 (Linux/macOS x86-64)

This is the calling convention you're likely using right now.

#### Argument Passing

- First six integer/pointer arguments: rdi, rsi, rdx, rcx, r8, r9 — in that order.

- First eight floating-point arguments: xmm0 through xmm7.

- Additional arguments: pushed onto the stack right-to-left.

- Aggregates (structs) larger than 16 bytes: passed via hidden pointer to a caller-provided slot.

#### Return Value

- Integer/pointer return: rax (and rdx if 16 bytes).

- Floating-point return: xmm0 (and xmm1 if needed).

- Larger aggregates: caller passes a hidden pointer; callee fills it.

#### Register Preservation

- Caller-saved ("scratch"): rax, rcx, rdx, rsi, rdi, r8-r11. Caller must save before calling if needed.

- Callee-saved ("preserved"): rbx, rbp, r12-r15. Callee must save and restore.

## 9.3 Why This Matters For Pointers

When you pass a pointer to a function, you're putting an 8-byte address into rdi (or wherever the convention says). The function reads that address and dereferences it. That's it — pointers travel through registers like any other 64-bit value.

// C source: void modify(int *p) { *p = 42; } int main(void) { int x = 0; modify(&x); return x; } // Generated assembly (x86-64, simplified): modify: movl $42, (%rdi) ; write 42 to address in rdi ret main: sub $24, %rsp movl $0, 12(%rsp) ; x at rsp+12 leaq 12(%rsp), %rdi ; load address of x into rdi call modify movl 12(%rsp), %eax ; load x for return add $24, %rsp ret

The interesting instruction is `leaq 12(%rsp), %rdi` — "load effective address." It computes rsp+12 (the address of x) and stores it in rdi. That IS the address-of operator at the machine level.

## 9.4 Variadic Functions and printf

Functions like printf take a variable number of arguments. The convention has to handle this. On x86-64 SysV:

- Named arguments use registers as usual (rdi for the format string).

- Variable arguments after the named ones go into registers (then stack) too.

- The callee uses the format string to figure out how many args to read and from where.

- If you give the wrong format specifier (%d for a string, etc.), the callee reads the wrong register and gets garbage — silent UB.

⚠ DANGER This is why format-string mismatches are dangerous. printf("%s", 42) treats the integer 42 as a pointer and tries to dereference it. The compiler can warn (-Wformat) but it's still a runtime hazard. Always enable format-string checking.

## 9.5 Other Conventions You'll Encounter

| Convention | Platform | Argument passing |
| --- | --- | --- |
| System V AMD64 | Linux/macOS x86-64 | Up to 6 ints in registers |
| Microsoft x64 | Windows x86-64 | Up to 4 ints in rcx, rdx, r8, r9 |
| cdecl | Old 32-bit x86 (Linux/Win) | All args on the stack, caller cleans up |
| stdcall | Win32 API on 32-bit | Stack, but callee cleans up |
| AAPCS | ARM/ARM64 | Up to 4 (or 8) args in r0-r3 (x0-x7) |

# Chapter 10 — The Heap, Inside

## 10.1 What Lies Beneath malloc

malloc() doesn't go to the OS for every allocation — that would be far too slow. Instead, the C library maintains a pool of memory in user space and serves small allocations from it. When the pool runs low, it asks the OS for more.

Two main system calls expand the pool:

- brk/sbrk — moves the "program break" higher to extend the data segment. Old, simple, used for small allocations.

- mmap — maps in anonymous pages from the OS. Used for large allocations (typically > 128 KB).

## 10.2 The Allocator's Bookkeeping

When you call malloc(N), the allocator:

- Checks its free list for a block of size ≥ N + sizeof(header).

- If it finds one, it splits the block (if it's much bigger) and gives you the data portion.

- If it doesn't, it asks the OS for more memory and adds it to the pool.

- It writes a header just before the address it returns to you, recording the block's size and a magic number.

- It returns the address of the data portion (just after the header).

| Address | Contents | Variable |
| --- | --- | --- |
| 0x600000 | size=24, flags | header (you don't see this) |
| 0x600010 | your byte 0 | ← what malloc returns |
| 0x600011 | your byte 1 | your data area |
| 0x600012 | ... | ... |
| 0x600027 | your byte 23 | (24 bytes total) |
| 0x600028 | size, flags | next block's header |

When you call free(p), the allocator:

- Looks at p - sizeof(header) to find your block's header.

- Checks the magic number to detect corruption.

- Marks the block as free in its free list.

- Optionally coalesces with adjacent free blocks to reduce fragmentation.

🔬 DEEPER This is why free(p) takes only a pointer — no size needed. The allocator stored the size in the header. It's also why double-free corrupts the heap: the second free reads garbage from the (now-freed) block, mangles the free list, and the next malloc may return overlapping or invalid memory.

## 10.3 Why malloc Can Fail

On 64-bit systems with virtual memory, malloc rarely returns NULL — the address space is essentially unlimited. But it CAN fail:

- Out of swap + RAM (overcommit can be disabled or exhausted).

- Process resource limits hit (ulimit -v).

- Address space fragmentation (rare on 64-bit, common on 32-bit).

- Asking for absurdly large amounts (size_t overflow if size * count overflows).

Always check. ALWAYS:

int *p = malloc(n * sizeof(int)); if (p == NULL) { perror("malloc"); // prints why return -1; // bail out gracefully }

## 10.4 Memory Fragmentation

After many allocations and frees of varying sizes, the heap can end up like Swiss cheese: enough total free space, but no single contiguous block large enough for your next request. This is fragmentation.

Modern allocators (jemalloc, tcmalloc, mimalloc) fight fragmentation aggressively with size classes, thread-local caches, and background coalescing. Glibc's allocator (ptmalloc2) is reasonable for most workloads.

💡 INSIGHT If your program does many allocations of the same small size, consider a pool allocator (one big malloc, then sub-allocate from it). Pool allocators trade flexibility for speed — they can be 10-100x faster than malloc/free for tight loops.

## 10.5 The valgrind / ASan Mental Model

Memory checkers work by tracking every byte's state. They maintain shadow memory recording:

- Whether each byte is allocated.

- Whether each byte has been initialized.

- Whether each byte is currently a redzone (out-of-bounds guard).

Every read/write/malloc/free updates the shadow. Any access to an unallocated, uninitialized, or redzone byte triggers a report. This is why these tools catch bugs that would otherwise be silent — they have ground truth about every byte's state.

# Chapter 11 — Alignment and Padding

## 11.1 What Alignment Is

CPUs prefer (and sometimes require) that multi-byte values live at addresses that are multiples of their size:

- A 1-byte char can live at any address.

- A 2-byte short should live at an even address.

- A 4-byte int should live at an address divisible by 4.

- An 8-byte long/double should live at an address divisible by 8.

- A 16-byte SIMD value should live at an address divisible by 16.

On x86-64, misaligned access is allowed but slower (the CPU may issue two memory transactions). On many ARM and RISC-V systems, misaligned access throws a hardware exception.

## 11.2 Why Structs Have Padding

struct S { char c; // 1 byte int i; // 4 bytes char d; // 1 byte }; // You might expect sizeof(struct S) == 6. // Actually, sizeof(struct S) == 12. // Why? Padding for alignment.

Memory layout:

| Address | Contents | Variable |
| --- | --- | --- |
| +0 | c (1 byte) | char |
| +1 | padding (3 bytes) | to align next int |
| +4 | i (4 bytes) | int |
| +8 | d (1 byte) | char |
| +9 | padding (3 bytes) | to make total a multiple of 4 |

The struct's size and alignment are determined by its largest member. Here, the struct must be 4-byte aligned (because of int i). And its size must be a multiple of 4 (so that an array of S keeps each element aligned). Hence the trailing padding.

## 11.3 Reordering For Smaller Structs

// Bad — 12 bytes: struct Bad { char c; int i; char d; }; // Good — 8 bytes: struct Good { int i; // +0..3 char c; // +4 char d; // +5 // +6..7 padding };

General rule: declare members in order of decreasing size. This minimizes padding.

## 11.4 Pointers and Alignment

This bites when you cast pointers between types:

char buf[8]; int *p = (int*)buf; // is buf 4-byte aligned? Maybe not. *p = 42; // potentially UB on strict-alignment archs // Safe approach: use memcpy int value = 42; memcpy(buf, &value, sizeof(value)); // alignment-safe

⚠ DANGER Casting a char* to int* and dereferencing it is undefined behavior unless the original char* is properly aligned for int. On x86 you'll usually get away with it; on ARM you'll get a bus error. memcpy with the right size is always safe.

## 11.5 alignof and alignas (C11)

#include <stdalign.h> alignof(int) // = 4 on most platforms alignof(double) // = 8 alignof(struct S) // = max alignment of any member // Force alignment for SIMD or cache-line reasons: alignas(64) char cacheline[64]; // 64-byte aligned // Useful with posix_memalign for aligned heap allocs: void *p; posix_memalign(&p, 64, 1024); // 64-byte aligned, 1 KB

# Chapter 12 — Aliasing and the Strict Aliasing Rule

## 12.1 What Aliasing Means

Two pointers alias if they point to the same memory location (or overlapping locations). Aliasing is normal:

int x = 5; int *p = &x; int *q = &x; // p and q alias *p = 10; printf("%d\n", *q); // prints 10 — they refer to the same int

## 12.2 The Compiler's Optimization Problem

Aliasing is a problem for optimizers. Consider:

void copy(int *dst, int *src, int n) { for (int i = 0; i < n; i++) { dst[i] = src[i]; } } // Could the compiler rearrange this loop, vectorize it, etc.? // ONLY if it knows dst and src don't overlap. // If they overlap (alias), the loop's semantics depend on order.

## 12.3 The Strict Aliasing Rule

To enable optimization, C imposes restrictions on what types of pointers may legally alias the same memory:

STRICT ALIASING (C17 §6.5) An object's stored value can only be accessed through an lvalue expression of the object's type, a compatible type, a qualified version of either, a signed/unsigned variant, an aggregate or union containing such a type, OR a character type (char, signed char, unsigned char). Anything else is undefined.

In plain English:

- You may access memory through a pointer of the same type.

- You may access memory of any type through a char* (this is the universal escape hatch).

- You may NOT, in general, access an int through a float*, or a struct A* through a struct B*.

## 12.4 The Classic Bug

// "Type punning" — interpreting an int's bytes as a float int i = 0x40490FDB; float f = *(float*)&i; // STRICT ALIASING VIOLATION // Compiler may optimize as if i is unchanged. // Correct way: memcpy float f; memcpy(&f, &i, sizeof(f)); // always allowed // Also correct: union union { int i; float f; } u; u.i = 0x40490FDB; float f = u.f; // C11 explicitly allows this

🔬 DEEPER GCC's `-fstrict-aliasing` (default at -O2) enables aggressive optimization based on the rule. If you violate it, the compiler may cache values in registers across stores it 'knows' don't alias — and your program produces wrong results, not a crash. Pass `-fno-strict-aliasing` to disable, but the right answer is to write code that respects the rule.

## 12.5 The restrict Keyword (C99)

Sometimes you KNOW two pointers don't alias, and you want the compiler to know too. That's what restrict promises:

void copy(int * restrict dst, const int * restrict src, int n) { for (int i = 0; i < n; i++) dst[i] = src[i]; } // "I promise: nothing read through src will be written through dst, // and vice versa, for the lifetime of these pointers." // // The compiler can now vectorize the loop, reorder loads/stores, // and produce dramatically faster code. // If you LIE — pass overlapping pointers — UB. // memcpy uses restrict; that's why memmove exists for overlap.

# Chapter 13 — void*, Type Punning, Generic Code

## 13.1 What void* Is

void* is a generic pointer — a pointer to bytes, with no information about what those bytes mean. It exists for two reasons:

- To pass pointers through interfaces that don't know the data's type (malloc, qsort, generic data structures).

- To convert between unrelated pointer types via an intermediate that the standard explicitly allows.

You CAN'T dereference void* — the compiler doesn't know how many bytes to read or how to interpret them. You CAN'T do pointer arithmetic on void* (technically; GCC allows it as an extension by treating it like char*).

## 13.2 Implicit Conversion to and from void*

int x = 5; int *p = &x; void *vp = p; // OK — implicit conversion int *q = vp; // OK — implicit back (C only; C++ requires cast) // malloc returns void*: int *arr = malloc(10 * sizeof(int)); // ↑ no cast needed in C // In C++, you'd need: // int *arr = (int*)malloc(...); // or better: new int[10];

💡 INSIGHT Do NOT cast malloc's return in C. The cast can hide bugs (forgetting to #include <stdlib.h> made malloc default to int return type, and a cast would silently mask the warning). The implicit conversion is correct and safer.

## 13.3 Generic Functions With void*

Standard library functions like qsort and bsearch are generic — they work with any data type — by taking void* arguments and a comparator function:

void qsort(void *base, size_t n, size_t size, int (*cmp)(const void *, const void *)); // To use with int: int cmp_int(const void *a, const void *b) { int x = *(const int*)a; int y = *(const int*)b; return (x > y) - (x < y); } int arr[5] = {3, 1, 4, 1, 5}; qsort(arr, 5, sizeof(int), cmp_int); // To use with strings: int cmp_str(const void *a, const void *b) { return strcmp(*(const char**)a, *(const char**)b); } const char *names[] = {"alice", "bob", "carol"}; qsort(names, 3, sizeof(char*), cmp_str);

## 13.4 Building Generic Data Structures

// A generic linked list node typedef struct Node { void *data; struct Node *next; } Node; Node *make_node(void *d) { Node *n = malloc(sizeof(Node)); if (n) { n->data = d; n->next = NULL; } return n; } // Use with any data: int x = 42; Node *n1 = make_node(&x); char *s = strdup("hello"); Node *n2 = make_node(s); // On retrieval, caller must know the type: int *xp = (int*)n1->data; // OK if you remember it's an int char *sp = (char*)n2->data;

⚠ DANGER Generic code via void* sacrifices type safety. The compiler can't catch type mismatches between insertion and retrieval. Modern alternatives use type-generic macros (C11 _Generic) or code generation, but void* remains the standard tool in C.

## 13.5 Type Punning — Reinterpreting Bytes

Sometimes you really do need to look at the bit-pattern of a value as a different type — for example, examining the bits of a float, or building a hashtable key from arbitrary data. The legitimate ways:

// Method 1: memcpy (always works, often optimizes to nothing) float f = 3.14f; uint32_t bits; memcpy(&bits, &f, sizeof(bits)); // Method 2: union (C11 guarantees this works) union { float f; uint32_t u; } pun; pun.f = 3.14f; uint32_t bits = pun.u; // Method 3: char* aliasing (always allowed) unsigned char *bytes = (unsigned char*)&f; // bytes[0..3] are the bytes of f

# Chapter 14 — Function Pointers and the ABI

## 14.1 What a Function Pointer Holds

A function pointer holds the address of the first instruction of a function in the code segment. When you call through it, the CPU jumps to that address.

int add(int a, int b) { return a + b; } int (*fp)(int, int) = add; int r = fp(3, 4); // 7 // At the assembly level: // 1. Place 3 in rdi, 4 in rsi (System V calling convention) // 2. CALL [fp] — indirect call through the address in fp // 3. Result is in rax

## 14.2 Function Pointers and Type Safety

Calling a function through a wrongly-typed pointer is undefined behavior. The compiler emits code based on the pointer's declared type — if reality differs, register/stack layout doesn't match:

int add(int a, int b) { return a + b; } void (*fp)(double) = (void (*)(double))add; fp(3.14); // UB // The compiler puts 3.14 in xmm0 (per the cast type). // But add reads its first arg from rdi (an int). // Garbage in, garbage out.

## 14.3 Function Pointers as Arrays — Dispatch Tables

int add(int a, int b) { return a + b; } int sub(int a, int b) { return a - b; } int mul(int a, int b) { return a * b; } int divv(int a, int b) { return a / b; } // Array of function pointers — a dispatch table int (*ops[])(int, int) = { add, sub, mul, divv }; // Call by index: int result = ops[op_code](x, y); // This is how interpreters dispatch opcodes, // how state machines transition, // and how virtual functions in C++ work under the hood.

## 14.4 Building Polymorphism in C

C has no classes, but you can simulate them with structs of function pointers — exactly how the Linux kernel and GTK+ are written:

typedef struct { void (*draw)(void *self); void (*free)(void *self); } ShapeOps; typedef struct { const ShapeOps *ops; // virtual table int x, y; } Shape; typedef struct { Shape base; int radius; } Circle; void circle_draw(void *self) { Circle *c = self; printf("Circle at (%d,%d) r=%d\n", c->base.x, c->base.y, c->radius); } void circle_free(void *self) { free(self); } const ShapeOps circle_ops = { circle_draw, circle_free }; Circle *new_circle(int x, int y, int r) { Circle *c = malloc(sizeof(*c)); c->base.ops = &circle_ops; c->base.x = x; c->base.y = y; c->radius = r; return c; } // Generic use: Shape *s = (Shape*)new_circle(10, 20, 5); s->ops->draw(s); // dispatches via vtable s->ops->free(s);

🔬 DEEPER This pattern — struct + vtable + 'methods' that take a void* self — is C's classic OOP idiom. Once you see it, you'll spot it everywhere: Linux's `struct file_operations`, OpenSSL's BIO methods, sqlite's vfs module. C++ classes compile to roughly this form.

## 14.5 Function Pointers in Signal Handlers and Threads

#include <signal.h> #include <pthread.h> void handler(int sig) { write(1, "got signal\n", 11); } signal(SIGINT, handler); // register function pointer void *worker(void *arg) { // ... return NULL; } pthread_t tid; pthread_create(&tid, NULL, worker, NULL); // worker is a function ptr

# Chapter 15 — Undefined Behavior: A Field Guide

## 15.1 What UB Really Means

Undefined behavior is not 'whatever the CPU happens to do.' It's 'the standard places no requirements on the result.' The compiler is free to assume your code does not invoke UB — and to optimize aggressively under that assumption.

This is critical. When a compiler sees a path that would only matter if UB occurred, it can prune that path. Your defensive code disappears. Your bounds check vanishes. Your null check evaporates.

THE COMPILER'S WORLDVIEW Modern compilers treat UB as a license to assume it doesn't happen. If your code says 'if (p->x > 0) ...' the compiler concludes p is non-null (because *p in p->x would be UB if p were null). It may then eliminate any later 'if (p == NULL)' check as dead code. Your safety check is gone — and you'll never see a warning.

## 15.2 The Pointer-Specific UBs

Memorize this list. These are the pointer operations that invoke undefined behavior:

- Dereferencing a NULL pointer.

- Dereferencing an uninitialized pointer.

- Dereferencing a pointer to freed memory (use-after-free).

- Dereferencing a pointer past an object's end (or before its beginning).

- Pointer arithmetic that produces a result outside an array (more than one past the end, or before the start).

- Comparing pointers (with <, <=, >, >=) that don't point into the same array.

- Subtracting pointers from different arrays.

- Calling free() on a pointer that wasn't returned by malloc/calloc/realloc — including stack/static addresses, or already-freed pointers (double-free).

- Calling free() on a pointer that has been incremented/decremented from the original malloc return value.

- Accessing memory through a pointer of the wrong type (strict aliasing violation).

- Misaligned access through a pointer.

- Modifying a string literal through a char*.

- Calling through a function pointer with a mismatched signature.

- Reading from uninitialized memory (e.g., from malloc but never written).

## 15.3 Spotting UB in Real Code

#### Example 1: The Null Check That Vanished

int *p = get_pointer(); int x = *p; // dereference — UB if p is NULL if (p == NULL) { // compiler may delete this — *p above return -1; // 'proved' p != NULL } return x; // Lesson: check FIRST, then dereference.

#### Example 2: The Off-By-One Loop

int arr[10]; for (int i = 0; i <= 10; i++) { // <= instead of < arr[i] = i; // arr[10] is past end — UB } // Optimizer may unroll this assuming i never exceeds 10 // — but the write at i=10 is UB regardless.

#### Example 3: The Returned Local

int *make(void) { int x = 42; return &x; // x dies on return — caller's ptr is dangling } int *p = make(); printf("%d\n", *p); // UB — reads from defunct stack frame

#### Example 4: The Strict Aliasing Violation

uint32_t i = 0x12345678; uint16_t *p = (uint16_t*)&i; // Cast OK uint16_t lower = *p; // STRICT ALIASING VIOLATION // (unless uint16_t and uint32_t // are the same type) // Use memcpy for type punning instead.

#### Example 5: The Use-After-Free

char *p = malloc(100); strcpy(p, "hello"); free(p); printf("%s\n", p); // UB — memory may have been recycled // Optimizer may NOT 'see' this is bad. // Result: garbage output, or crash, or // accidentally working today.

## 15.4 Tools Against UB

UB is sneaky. The defensive arsenal:

| Tool | Catches | Cost |
| --- | --- | --- |
| gcc -Wall -Wextra | Many simple UB at compile time | None |
| clang -fsanitize=address | Memory UB at runtime | ~2x slowdown |
| clang -fsanitize=undefined | Other UB at runtime | Small |
| Valgrind | Memory UB at runtime | 10-50x slowdown |
| scan-build (clang static analyzer) | Many UB without running | Compile-time only |
| Coverity / PVS-Studio | Lots of UB and bugs | Commercial |

💡 INSIGHT Make `gcc -Wall -Wextra -Wpedantic -fsanitize=address,undefined -g -O1` your daily development build. Switch to `-O2` (without sanitizers) only for release. The sanitizers will catch UB the moment it happens — making intermittent bugs reproducible.

## 15.5 Why You Can Sometimes 'Get Away With' UB

Beginners often run UB-laden code and see it 'work.' They conclude the warnings are paranoia. They're not.

UB code may work because:

- The optimizer didn't notice the UB this time.

- The recycled memory happened to still hold valid-looking data.

- The off-by-one wrote into padding bytes that nothing else uses.

- The misaligned access happens to work on x86 (just slower).

Then a compiler upgrade, an optimization-flag change, or a different platform makes the UB manifest. The bug appears 'out of nowhere.' Production crashes. Customers leave. UB is technical debt with compound interest.

# Chapter 16 — Mastery Drills

Fifty problems. Solve them on paper before running. Each tests a specific concept covered in this volume. Don't skip — these are the exercises that turn understanding into mastery.

## 16.1 Predict the Output (Problems 1–20)

For each problem, predict the output. Then run it. If your prediction was wrong, review the relevant chapter.

#### Problem 1

int x = 5; int *p = &x; *p = 10; printf("%d\n", x);

Expected: 10

#### Problem 2

int a = 1, b = 2; int *p = &a, *q = &b; int *t = p; p = q; q = t; printf("%d %d\n", *p, *q);

Expected: 2 1

#### Problem 3

int a = 1, b = 2; int *p = &a, *q = &b; *p = *q; printf("%d %d\n", a, b);

Expected: 2 2

#### Problem 4

int arr[3] = {10, 20, 30}; int *p = arr; printf("%d %d\n", *p, *(p+2));

Expected: 10 30

#### Problem 5

int arr[3] = {10, 20, 30}; printf("%d %d\n", arr[1], 1[arr]);

Expected: 20 20

#### Problem 6

char s[] = "abc"; printf("%zu\n", sizeof(s));

Expected: 4 (chars + null)

#### Problem 7

char *s = "abc"; printf("%zu\n", sizeof(s));

Expected: 8 (pointer)

#### Problem 8

int x = 5; int *p = &x; int **pp = &p; printf("%d\n", **pp);

Expected: 5

#### Problem 9

int arr[3] = {1, 2, 3}; int *p = arr + 1; printf("%d\n", p[-1]);

Expected: 1

#### Problem 10

int a[5] = {0}; int *p = a; int *q = a + 5; printf("%td\n", q - p);

Expected: 5

#### Problem 11

char *s = "hello"; printf("%c\n", *(s+1));

Expected: e

#### Problem 12

int x = 5, y = 10; int *arr[2] = {&x, &y}; printf("%d\n", *arr[1]);

Expected: 10

#### Problem 13

int x = 0; int *p = &x; *p = *p + 1; printf("%d\n", x);

Expected: 1

#### Problem 14

int arr[3] = {10, 20, 30}; int *p = arr; printf("%d\n", *p++);

Expected: 10 (p now at arr[1])

#### Problem 15

int arr[3] = {10, 20, 30}; int *p = arr; printf("%d\n", *++p);

Expected: 20

#### Problem 16

int arr[3] = {10, 20, 30}; int *p = arr; (*p)++; printf("%d\n", arr[0]);

Expected: 11

#### Problem 17

char s[] = "hello"; char *p = s; while (*p) p++; printf("%td\n", p - s);

Expected: 5

#### Problem 18

int n = 3; int *p = malloc(n * sizeof(int)); for (int i = 0; i < n; i++) p[i] = i*i; printf("%d\n", p[2]); free(p);

Expected: 4

#### Problem 19

int x = 0x12345678; char *p = (char*)&x; printf("%x %x\n", p[0], p[3]);

Expected: 78 12 (little-endian) OR 12 78 (big-endian)

#### Problem 20

int arr[2][3] = {{1,2,3},{4,5,6}}; int *p = &arr[0][0]; printf("%d\n", p[4]);

Expected: 5

## 16.2 Find the Bug (Problems 21–35)

Each snippet has at least one bug. Identify it and explain why it's a bug. Many are UB.

#### Problem 21

int *p; *p = 5;

Bug: p is uninitialized — *p writes to a random address. UB.

#### Problem 22

char *get(void) { char buf[100]; strcpy(buf, "hi"); return buf; }

Bug: Returns pointer to local buf — dangling after return. UB.

#### Problem 23

int *p = malloc(sizeof(int)); *p = 5; free(p); printf("%d\n", *p);

Bug: Use-after-free. UB.

#### Problem 24

char *p = "hello"; p[0] = 'H';

Bug: Modifying string literal — read-only memory. UB. Crash on most systems.

#### Problem 25

int arr[10]; for (int i = 0; i <= 10; i++) arr[i] = i;

Bug: arr[10] is one past the end. Writing UB.

#### Problem 26

int *p = malloc(100 * sizeof(int)); p++; free(p);

Bug: free() with non-malloc'd pointer. Even if p was returned by malloc, modifying it before free is UB.

#### Problem 27

int *p = malloc(sizeof(int)); free(p); free(p);

Bug: Double free. UB.

#### Problem 28

int x = 5; int *p = &x; free(p);

Bug: free() on stack address. UB.

#### Problem 29

int *p = malloc(10 * sizeof(int)); if (p < NULL) return;

Bug: Comparison of unrelated pointers (p vs NULL with <) is UB. Use p == NULL.

#### Problem 30

void copy(int *a, int *b, int n) { for (int i = 0; i < n; i++) a[i] = b[i]; } int x[10] = {1,2,3,4,5}; copy(x+1, x, 5);

Bug: Overlapping copy with no restrict guard — but if a and b alias, the copy reads what it just wrote. Use memmove for overlap.

#### Problem 31

int x = 5; float *fp = (float*)&x; printf("%f\n", *fp);

Bug: Strict aliasing violation. Use memcpy for type punning.

#### Problem 32

char buf[3]; strcpy(buf, "hello");

Bug: Buffer overflow — buf has room for 2 chars + null, not 5+null. UB and security bug.

#### Problem 33

int a = 5, b = 3; int *p = &a; ptrdiff_t d = &b - &a;

Bug: Pointer subtraction across different objects. UB.

#### Problem 34

int *p = NULL; if (*p == 0) puts("null");

Bug: Dereferencing NULL. UB. Test p == NULL, not *p.

#### Problem 35

int *p = malloc(sizeof(int)); p = realloc(p, 100 * sizeof(int)); if (!p) return -1;

Bug: If realloc fails, original p is leaked AND we lost the pointer to free it. Assign to a temporary, check, then update.

## 16.3 Implementation Challenges (Problems 36–50)

Implement each from scratch. No standard library beyond stdio (and stdlib for malloc-related ones). Use sanitizers when testing.

- 36. Implement strlen using only pointer arithmetic (no [] indexing).

- 37. Implement strcpy. Make sure to return the correct value.

- 38. Implement strcmp. Return correct sign.

- 39. Implement strcat (concatenate; assume dest is large enough).

- 40. Implement memcpy with no overlap requirement.

- 41. Implement memmove (handles overlap correctly).

- 42. Implement strdup using malloc.

- 43. Implement a function `int *make_int(int v)` that allocates and returns a pointer to an int holding v.

- 44. Implement a function `int **make_2d(int rows, int cols)` that allocates a 2D matrix on the heap, with proper cleanup on partial failure.

- 45. Implement `void free_2d(int **m, int rows)` that correctly frees the above.

- 46. Implement a generic swap: `void swap(void *a, void *b, size_t size)` using memcpy.

- 47. Implement a singly-linked list of ints with: push_front, pop_front, length, free_all.

- 48. Implement the linked-list delete using pointer-to-pointer (eliminating the head special case).

- 49. Implement bsearch's signature: `void *bsearch(const void *key, const void *base, size_t n, size_t size, int (*cmp)(const void*, const void*))`. Test with int and string arrays.

- 50. Implement a tiny memory pool: `pool_create(size)`, `pool_alloc(pool, n)`, `pool_destroy(pool)`. Track allocations in a free list. Use sanitizers to verify zero leaks and zero out-of-bounds.

If you can do all 50 without help, you have mastered pointers. Genuinely.

# Appendix A — Quick Reference

## A.1 Pointer Operators

| Operator | Meaning | Returns |
| --- | --- | --- |
| &x | Address of x | Pointer to x |
| *p | Dereference p | Object at address p |
| p->m | Member m via p | (*p).m |
| p[i] | Element at offset i | *(p + i) |
| p + n | Pointer arithmetic | Address n elements forward |
| p - q | Pointer difference | Number of elements between |

## A.2 Pointer Declaration Templates

int *p; // pointer to int const int *p; // pointer to const int (can't write *p) int *const p; // const pointer to int (can't reassign p) const int *const p; // const pointer to const int (neither) int *p[10]; // array of 10 int pointers int (*p)[10]; // pointer to array of 10 ints int *f(int); // function returning int* int (*f)(int); // pointer to function returning int int *(*f)(int); // pointer to function returning int* int (*f[10])(int); // array of 10 function pointers

## A.3 The Cardinal Rules

- Always initialize pointers (NULL or a valid address).

- Always check malloc/calloc/realloc for NULL.

- Always pair every malloc with exactly one free.

- Always set pointer to NULL after free.

- Never return a pointer to a local variable.

- Never modify a string literal through char*.

- Never go past array bounds — even just to compute, more than one past is UB.

- Never compare pointers from different arrays with < <= > >=.

- Never subtract pointers from different arrays.

- Never type-pun via cast — use memcpy or union.

- Never call free on a non-malloc'd pointer or one already freed.

- Never trust that 'it works on my machine' means it's correct.

## A.4 Reading List, In Order

- Kernighan & Ritchie, "The C Programming Language" — Chapters 5, 6, 7. The classic. Short, dense, perfect.

- Reek, "Pointers on C" — devoted entirely to this topic.

- Reese, "Understanding and Using C Pointers" — modern and thorough.

- van der Linden, "Expert C Programming: Deep C Secrets" — informal but full of insight.

- Bryant & O'Hallaron, "Computer Systems: A Programmer's Perspective" — Chapters 3, 6, 9. Connects pointers to assembly, virtual memory, and the OS.

- Seacord, "Secure Coding in C and C++" — what attackers do with pointer mistakes.

- ISO/IEC 9899:2018 (C17 standard) — when nothing else has the precise answer. Available as a draft (N2310).

— End of Volume II —

Pointers stop being mysterious when you understand the layers below them.

Now go write code that earns its memory.